Initially, I tried to run the simulation environment directly on macOS. However, installing and configuring ROS 2, Gazebo, and Nav2 on a MacBook Air M2 was difficult because of the ARM architecture and compatibility issues. To improve stability and development speed, I installed VMware Fusion Pro and set up an Ubuntu 24.04.4 Desktop ARM64 virtual machine.

After setting up the Ubuntu environment, I installed ROS 2 Jazzy, Gazebo, and Nav2. I first tested the standard ROS 2 Nav2 TurtleBot simulation in Gazebo and RViz to understand how Nav2 performs path planning and obstacle avoidance before applying it to the drone scenario. The TurtleBot demo was useful because it already provides the interfaces expected by Nav2, including odometry, transforms, laser scan data, costmaps, and velocity commands.

In the TurtleBot demo, I initialized the robot position in RViz using “2D Pose Estimate” and then sent navigation goals using “Nav2 Goal.” Nav2 successfully planned paths around obstacles and controlled the robot to reach the selected goals. I also observed that the robot sometimes moved backwards or rotated to avoid obstacles, showing that Nav2 uses its local controller and costmap rather than simply moving directly toward the target.

This confirmed that the Nav2 navigation pipeline works correctly in my Ubuntu VM setup. It also provided a baseline for adapting Nav2 to the drone simulation.

## Required Bridges for Applying Nav2 to the Drone Simulation

Nav2 cannot directly control the drones in the current Gazebo setup because the drones are not yet exposed to ROS 2 in the same way as TurtleBot. The current drone simulation is mainly controlled through Gazebo topics and Python scripts, while Nav2 expects ROS 2 topics and coordinate frames.

To apply Nav2 to the drones, several bridges are required. First, Nav2 publishes velocity commands on /cmd_vel using geometry_msgs/msg/Twist. These commands must be converted into the Gazebo command topic used by the drone. Second, Nav2 needs odometry and transform information, so the drone pose from Gazebo must be converted into /odom and /tf. Third, Nav2 needs obstacle information, either from a static map, a simulated sensor such as lidar, or directly published obstacle positions.

For the first prototype, I simplified the problem by using only one drone at a fixed altitude. In this setup, Nav2 can be treated as a two-dimensional planner in the horizontal x-y plane, while the drone maintains a constant height.

The intended integration pipeline is:

Gazebo drone pose → ROS 2 /odom and /tf → Nav2 localization and planning
Nav2 /cmd_vel → ROS-Gazebo bridge → Gazebo drone velocity command → drone movement

## First Bridge Between ROS 2 and the Gazebo Drone

To begin the integration, I created a new SDF world containing one X3 UAV, a ground plane, and one static obstacle. Using one drone instead of the previous three-drone collision scenario made the first Nav2 integration easier to debug.

The drone model was loaded from the locally downloaded Gazebo Fuel model directory, and the X3 UAV multicopter motor and velocity-control plugins were kept in the SDF. The drone was started at a fixed altitude so that the first prototype could focus on horizontal x-y movement.

I first created a Python bridge script to subscribe to ROS 2 /cmd_vel, publish odometry and transform information, and send velocity commands to the drone. However, sending Gazebo commands from Python using repeated gz topic subprocess calls caused timeout warnings and was not reliable for continuous control.

To debug this, I inspected the available Gazebo topics for the drone and found the correct command topics:

/drone_1/enable
/drone_1/gazebo/command/motor_speed
/drone_1/gazebo/command/twist

An earlier attempt used /model/drone_1/gazebo/command/twist, but this was not the topic used by the drone controller. After identifying the correct topic, I tested direct Gazebo control by publishing gz.msgs.Twist messages to /drone_1/gazebo/command/twist. The drone moved successfully, confirming that the Gazebo-side controller was working.

Next, I replaced the subprocess-based command method with ros_gz_bridge. This bridge converted ROS 2 geometry_msgs/msg/Twist messages from /cmd_vel into Gazebo gz.msgs.Twist messages on /drone_1/gazebo/command/twist. After enabling the drone and publishing ROS 2 /cmd_vel commands, the drone moved successfully in Gazebo.

This completed the first bridge required for Nav2-drone integration:

ROS 2 /cmd_vel → ros_gz_bridge → Gazebo /drone_1/gazebo/command/twist → drone movement

This established the command side of the Nav2-drone interface.

## Second Bridge: Gazebo Drone Pose to ROS 2 Odometry and TF

After completing the first bridge from ROS 2 /cmd_vel to the Gazebo drone velocity command topic, I implemented the second required bridge in the opposite direction. The goal of this bridge was to publish the drone’s simulated pose from Gazebo into ROS 2 as /odom and /tf, so that Nav2 can later know where the drone is located.

For this step, I used Gazebo’s world pose information topic. Initially, I tested /world/drone_nav2_world/pose/info, but this topic contained pose information for several entities in the world, including the ground plane, obstacle, sun, and drone. I then switched to /world/drone_nav2_world/dynamic_pose/info, which is more suitable because it focuses on dynamic entities such as the drone.

The Gazebo dynamic pose topic was bridged into ROS 2 using ros_gz_bridge and remapped to /gz_pose. I then created a Python node called drone_pose_to_odom_tf.py. This node subscribes to /gz_pose, extracts the drone pose, and continuously publishes:

/odom
odom -> base_link transform
map -> odom static transform

During testing, the bridged Gazebo pose messages did not preserve clear frame names, so the script could not directly identify a frame named drone_1. To solve this, the Python node was modified to select the most suitable dynamic transform and publish it as the drone pose. Since the current test world contains only one drone, this was sufficient for the single-drone prototype.

After running the dynamic pose bridge and the Python odometry/TF bridge, the /odom topic appeared correctly in ROS 2, and tf2_echo confirmed that the odom -> base_link transform was being published. When the drone was moved using ROS 2 /cmd_vel, the odometry values changed accordingly. For example, the x-position increased from approximately 0.0 to values such as 0.27, 1.97, and 3.26, confirming that the ROS 2 odometry was tracking the drone movement.

This completed the second required bridge:

Gazebo drone pose → ros_gz_bridge → ROS 2 /gz_pose → Python node → /odom and /tf

At this point, the drone simulation has both basic interfaces needed for future Nav2 integration:

ROS 2 /cmd_vel → Gazebo drone movement
Gazebo drone pose → ROS 2 /odom and /tf

Nav2 needs both of them, the command output and the feedback input. The log shows that /odom is being published correctly and that the ROS 2–Gazebo bridge is working. The next step is to use these two bridges together and test whether Nav2 can send velocity commands to the drone based on its estimated position.

## Combined Bridge Test and Automation

After confirming the command bridge and the feedback bridge separately, I tested both bridges together. In this setup, ROS 2 velocity commands are sent to the drone through the /cmd_vel bridge, while the drone pose is simultaneously published back to ROS 2 through /odom and /tf.

To make the testing process easier and more repeatable, I created shell scripts to start and stop the bridge stack automatically. The startup script launches the Gazebo drone world, the Gazebo dynamic pose bridge, the Python odometry/TF bridge, the ROS 2 /cmd_vel to Gazebo command bridge, and the drone enable command. I also created a separate movement test script that publishes a forward /cmd_vel command for a few seconds and then sends zero velocity commands to stop the drone.

This automation reduces the number of manual terminal commands needed for each test and makes the setup more reliable for repeated experiments. After running the automated stack, the drone moved successfully in Gazebo, and the /odom values updated during the movement. This confirmed that the command and feedback bridges can operate together correctly.

The completed automated bridge pipeline is:

start_drone_stack.sh → launches Gazebo and all required bridges
test_move_drone.sh → publishes /cmd_vel and verifies drone movement
stop_drone_stack.sh → stops the Gazebo and bridge processes

In the next step Nav2 replaces the manual movement test script and generates /cmd_vel commands based on navigation goals.

## Third Bridge: Obstacle Sensing for Nav2

After the command and odometry bridges, the last missing input was obstacle information. Nav2 builds its costmaps from sensor data. Without a sensor the costmap stays empty and Nav2 plans straight through obstacles. I first added a gpu_lidar sensor to the drone in the SDF world, enabled the Gazebo sensors plugin, and bridged the scan topic into ROS 2 as /scan.

The sensor registered and its topic appeared in the Gazebo topic list, but it never published any data. The reason is that gpu_lidar computes its ranges on the GPU and needs a working OpenGL context, which my VMware virtual machine cannot provide reliably. Without 3D acceleration Gazebo reports "No 3D enabled". With 3D acceleration enabled, the virtual GPU driver is unstable: the Gazebo GUI hangs and the sensor still publishes nothing.

Since obstacle information can also come from directly published obstacle positions, I replaced the GPU lidar with a virtual lidar node, virtual_lidar.py. It subscribes to /odom, casts 360 rays from the drone pose against the known obstacle rectangles, and publishes a normal sensor_msgs/LaserScan on /scan at 10 Hz. For Nav2 the result looks like a simulated lidar and no rendering is needed. The obstacle list is kept in sync with the world file. On a machine with a working GPU, the real gpu_lidar can be used again.

## Improved Odometry, Simulation Time, and Cleanup

I replaced the height-based frame selection workaround in the odometry bridge. The drone model now uses Gazebo’s OdometryPublisher plugin, which publishes the true pose and velocity on its own topic /model/drone_1/odometry. The Python bridge subscribes to that topic and republishes /odom and the odom -> base_link transform. There is no more guessing which entity is the drone, and the velocity comes from the simulator instead of my own finite differences.

I also bridged Gazebo’s /clock into ROS 2 and enabled use_sim_time on all nodes, so everything shares one simulation clock. The old prototype bridge was moved out of the working folder, and every terminal and test run now writes its output into a logs/ folder. This logging turned out to be very useful for the debugging described below.

## Nav2 Configuration and First Obstacle Avoidance Runs

Nav2 runs without a map and without AMCL. The map -> odom transform is a static identity, and both costmaps are filled only from /scan (an obstacle layer plus an inflation layer). The DWB controller is configured as holonomic so the planner can also use sideways motion. The standard Jazzy nav2_bringup launch file starts extra servers (collision monitor, route server, docking server) that fail without their own configuration, so I wrote a minimal launch file with only the controller, planner, behavior server and BT navigator, plus a lifecycle manager.

The first test scenario is a 1x4x2 m wall at x = 4 and a goal at (8, 0), so a straight path would cross the obstacle. The first run succeeded in about 20 seconds with no recovery behaviors. The return flight to (0, 0) aborted after three minutes and 17 recoveries. The logs showed that the drone had actually reached a point 0.24 m from the goal within 20 seconds, which is inside the position tolerance, but it arrived facing away from the requested final heading. Rotating in place produces no translation, so Nav2’s progress checker kept reporting "failed to make progress" and finally gave up.

A quadcopter’s final heading does not matter because it can move in any direction, so I disabled the yaw goal tolerance and removed the rotate-to-goal critic from the controller. After this change, repeated back-and-forth runs completed 3 out of 3 with SUCCEEDED, about 18 to 19 seconds per 8 m leg, zero recoveries, and a ratio of distance traveled to Euclidean distance of about 1.3. This ratio is the effectiveness metric suggested for the evaluation, computed directly from the recorded logs.

## Visualization and Portability

I added two viewers that attach to the running simulation. view.sh opens RViz with a preconfigured display: the odometry trail (the actually flown path), the lidar points, the global costmap with its inflation band, and the planned path. Goals can also be sent by clicking in the view. view_gazebo.sh attaches Gazebo’s own 3D window to the headless server. In the virtual machine the Gazebo GUI only works with 3D acceleration disabled (software rendering is slow but stable). On machines with a real GPU it works normally.

I also removed the remaining portability problems. The world file now loads the X3 model through its Gazebo Fuel URI instead of a local absolute path, all scripts find their own directory instead of assuming a fixed one, and the repository README documents the requirements and the step-by-step procedure. The project now runs unchanged on other machines.

## Phase A: Random Scenarios and Hit Counting

For the testing protocol I wrote a scenario generator, gen_scenario.py. It takes the number of obstacles and a random seed, then picks a random start point, a random goal point at least 8 m away, and N random box obstacles with obstacle-free zones around the start and the goal. It writes both the Gazebo world (world.sdf, with the drone spawned at the random start) and a machine-readable scenario.yaml from the same data, so the simulation, the virtual lidar and the collision counter always agree about the obstacle positions. The same seed always produces the same scenario.

Collision counting follows the protocol: the run is not stopped on contact. A new node, hit_monitor.py, models the drone as a 0.3 m disc, counts each contact episode with an obstacle, and tracks the minimum clearance over the whole flight. Hits are counted from the moment the goal is sent until it finishes. Contacts that happen while the drone is parked (for example an obstacle drifting over it before the goal is sent) are excluded, since they are not navigation failures.

The first randomized runs exposed one more Nav2 default that does not fit a multicopter. Goals that required mostly sideways flight stalled, while forward goals worked. Recording /cmd_vel showed that Nav2 was sending correct velocity commands and that the drone ignored them. By replaying the commands manually I found the cause: any command with a sustained yaw rate stalls the X3 velocity controller completely, while pure translation works. I therefore removed the two heading alignment critics and set the controller to command no rotation at all (max_vel_theta: 0.0). The drone keeps its spawn heading and moves by strafing, which a quadcopter can do freely. After this fix the verification scenarios passed: 4 obstacles with a 12 m sideways path (SUCCEEDED in 32 s, no recoveries, no hits) and a 10-obstacle scenario (SUCCEEDED in 18 s, no hits, minimum clearance 0.59 m).

## How to Run (Current Procedure)

Each numbered step runs in its own terminal from the nav2/ folder:

./gen_scenario.py --n-obstacles 4 --seed 42 generates a scenario (into scenarios/s42_n4/).

./start_drone_stack.sh scenarios/s42_n4 starts Gazebo (headless), the bridges, the virtual lidar, the hit monitor and the obstacle mover. Wait until the odometry terminal prints Publishing /odom lines. Without the argument, the classic single-wall world is used.

./launch_nav2.sh starts Nav2. Wait for "Managed nodes are active".

./view.sh (optional) opens the RViz view: costmap, planned path, lidar points, odometry trail and the obstacle markers.

./view_gazebo.sh (optional) attaches the Gazebo 3D window. In the VM this requires 3D acceleration to be disabled.

./send_nav2_goal.sh --scenario scenarios/s42_n4 arms the drone and sends the scenario goal. ./send_nav2_goal.sh X Y sends an explicit goal instead.

cat logs/hits_current.yaml shows hits and minimum clearance. The full feedback stream of every goal is kept in logs/goal_<time>.txt.

./stop_drone_stack.sh stops everything (stop Nav2 with Ctrl+C in its terminal).

## Phase B: Batch Evaluation with Static Obstacles

To produce the statistics I wrote a batch runner, run_batch.sh. For every combination of obstacle count and seed it generates a scenario, starts the whole stack in the background (without terminal windows), waits until odometry and Nav2 are ready, arms the drone, sends the goal with a watchdog timeout, saves all logs and results, and shuts everything down before the next run. If a run fails, the batch records the failure and continues. A second script, compute_metrics.py, reads the recorded files and computes success rate, navigation time, the path to Euclidean distance ratio, hit count and minimum clearance per obstacle count, and creates the corresponding plots.

The main experiment swept 1, 2, 4, 6, 8 and 10 obstacles with 8 random seeds each, 48 runs in about 46 minutes without supervision. All 48 runs reached the goal with zero hits:

N   success   mean time   path/Euclid   hits   min clearance

1     8/8      29.3 s        0.980        0        0.80 m

2     8/8      29.3 s        0.981        0        0.76 m

4     8/8      29.3 s        0.986        0        0.52 m

6     8/8      30.6 s        1.001        0        0.38 m

8     8/8      31.5 s        1.019        0        0.37 m

10     8/8      31.6 s        1.021        0        0.37 m

The trends are as expected. With few obstacles the direct line is usually free, so the ratio is slightly below 1 (the drone stops 0.3 m before the goal point, inside the goal tolerance). With more obstacles the navigation time and the detour grow, and the minimum clearance drops toward the costmap inflation radius, which acts as the safety margin.

In the first attempts a few runs failed with timeout errors inside Nav2. The cause was the limited RAM of the virtual machine: when the VM ran out of memory, the simulation froze for a while and Nav2’s internal service calls timed out. These were infrastructure failures, not navigation failures. I increased a Nav2 timeout parameter (default_server_timeout) and repeated the affected runs with the same seeds and the same configuration, which gave the final 48 out of 48. The curated results (CSV, summary table, plots) are stored in the repository under results/2026-07-17_static_sweep/.

## Phase C: Moving Obstacles

The last part of the protocol repeats the evaluation with moving obstacles. The motion is implemented in the same virtual way as the sensing. A new node, obstacle_mover.py, moves the obstacle rectangles at a constant speed in random directions and lets them bounce off the workspace bounds. It publishes the current positions at 20 Hz, and the virtual lidar, the hit monitor and RViz all use this single source, so the drone always senses the obstacles where they currently are. Moving obstacles are not placed in the Gazebo world, because a physical box frozen at its spawn position could collide with the drone at a place where the obstacle virtually no longer is.

The sweep used 6 obstacles, speeds from 0.2 to 2.0 m/s, and 8 seeds per speed (48 runs):

v (m/s)   success   mean time   path/Euclid   hits

0.2        8/8       32 s         1.11         1

0.4        8/8       39 s         1.15        19

0.8        8/8       48 s         1.38         9

1.2        8/8       50 s         1.48        35

1.6        8/8       71 s         1.97        50

2.0        8/8       71 s         1.81        73

Because the runs are not stopped on contact, the drone always reaches the goal in the end, so the success column stays at 8/8. The degradation shows in the other columns. The hit count grows from 1 contact in eight flights at 0.2 m/s to 73 contacts (about nine per flight) at 2.0 m/s. The navigation time roughly doubles, and the flown path grows to about twice the straight distance. The explanation is that Nav2 only reacts to where obstacles currently are and knows nothing about their velocity. The drone’s own speed limit is about 0.6 to 0.7 m/s, so from roughly 0.8 m/s the obstacles are faster than the drone and reactive replanning cannot keep up.

This completes the evaluation protocol for Nav2. Nav2 handles static and slowly moving obstacles well, and gets much worse once the obstacle speed approaches the drone’s own speed. This is the baseline for the avoidance component that will be added on top of Nav2. The same scenario generator, batch runner and metrics can be reused later for the comparison with and without the component. The curated results are in results/2026-07-20_speed_sweep/.

## Moving to PX4 and MAVLink

After the professor’s feedback, I changed the architecture in two ways. First, communication with the simulated drone now goes through MAVLink, the protocol used between a real drone and its onboard software, instead of my own bridge. Second, the drone is now a PX4 vehicle, so the flight control software is the same one that runs on real hardware.

The setup has four parts. Gazebo simulates the physics. PX4 runs in software-in-the-loop mode and acts as the flight controller, keeping the drone stable and executing position commands. MAVROS translates between PX4’s MAVLink protocol and ROS 2. My own ROS 2 nodes decide where the drone should go and measure what happens.

I installed PX4 v1.17.0 and built it for simulation with the x500 quadcopter model. Two problems had to be solved before the drone would fly. PX4 refuses to arm when no ground control station is connected, so I wrote a small script that sends the minimal MAVLink heartbeat a ground station would send. PX4 also refused to arm because the simulated power monitoring reported unhealthy values, which is disabled with a parameter. After that the drone armed and took off, first from the PX4 console and then from ROS 2 through MAVROS.

The important step was offboard control. In offboard mode the drone follows position setpoints sent from outside, which is how a companion computer flies a real drone. PX4 requires setpoints to already be streaming at more than 2 Hz before it will accept the mode, so my node starts publishing first and only then requests the switch. With this the drone flew from ROS 2 to an arbitrary point in 3D, for example to (5, -4, 6), and held it to within a few centimetres. This is the control interface all the later experiments use.

## What PX4 Offers for 3D Navigation

The plan was to replace Nav2 with PX4’s own 3D path planning and collision avoidance. While setting this up I found that this software is no longer available. The PX4-Avoidance package, which contains the 3D local and global planners, was archived by the PX4 project in August 2024 and is marked as not maintained. It also only supports ROS 1 and Ubuntu 20.04, while my system uses ROS 2 and Ubuntu 24.04. The MAVLink path planning interface that connected an external planner to PX4 missions has also been withdrawn, and the current documentation says it should not be used in any PX4 version.

What PX4 still provides is Collision Prevention, which slows the drone before it reaches an obstacle. It works only in the horizontal plane and only in Position mode, so it cannot serve as a 3D navigation system for autonomous flight.

The approach PX4 recommends today is the one my setup already uses: the planning runs on the companion computer side in ROS 2, and PX4 executes the resulting setpoints in offboard mode. I told the professor about this before building the experiments on top of it. Since PX4 has no 3D avoidance of its own, the flights below are a baseline without any avoidance. The full note with sources is in the repository as px4/FEASIBILITY_3D_NAVIGATION.md.

## 3D Maze Scenarios and Collision Detection

For the 3D experiments I wrote a new scenario generator, gen_scenario_3d.py. It places n obstacles at random, each a box standing on the ground with a random footprint and a random height, which produces the maze described in the plan. It also places k drones at random, each with its own target in 3D space, and keeps a clear zone around every start and target so that no drone begins or ends inside an obstacle. Everything is written to a single scenario.yaml, and the same seed always reproduces the same scenario. The file already holds a list of drones, so the format does not need to change for the multi-drone phase.

The obstacles are inserted into the running Gazebo world at runtime rather than written into a world file. This keeps the PX4 installation untouched, and the same world file works for every scenario.

A second node, collision_monitor_3d.py, checks the drone against every obstacle. The drone is treated as a sphere and each obstacle as a box, and because the boxes have different heights the check is three-dimensional: the same horizontal position is safe above a low obstacle and a collision against a tall one. Following the plan, a drone that collides is destroyed rather than counted and allowed to continue.

Two details took some work. The scenario is written in Gazebo world coordinates, but MAVROS reports the drone in PX4’s local frame, whose origin is wherever the drone was spawned. The nodes therefore convert between the two by adding the start position. Without this the drone flies to a completely different place. The second detail is that PX4 refuses an ordinary disarm command while the drone is in the air, answering "Disarming denied: not landed". In one test this meant a drone survived its own collision, tumbled around the world and eventually reached its target after travelling 350 m instead of 15 m, which would have been meaningless data. Sending the disarm command with PX4’s force option solves it.

## Baseline Missions in 3D

The mission node, run_mission.py, flies a drone to its scenario target using offboard setpoints, with no avoidance of any kind. It climbs first and then heads for the target, records the flown path, and stops as soon as the drone either arrives or is destroyed. It writes the Euclidean distance between start and target, the distance actually travelled, their ratio, and the simulation time.

Both outcomes were verified. A mission through an 8-obstacle maze reached its target:

outcome: reached      euclidean 21.33 m   travelled 23.57 m

ratio: 1.105          time 11.6 s         min clearance 1.61 m

and a mission through a denser 30-obstacle maze was destroyed on the way:

outcome: destroyed    euclidean 15.13 m   travelled 8.50 m

collision at world (-7.49, -3.38, 5.68)   min clearance -0.067 m

The negative minimum clearance in the second run means the drone had entered the obstacle by about 7 cm when the collision was registered, and the recorded collision point matches the drone’s final position, so it stopped where it was hit. One point to remember for the evaluation: the ratio is only meaningful for drones that reach their target, because a drone destroyed halfway has travelled less than the straight-line distance and produces a ratio below 1. Averages will be taken over successful flights, with destroyed flights counted separately.

## Flying Several Drones at Once

To run k drones at the same time I wrote start_px4_multi.sh. It starts one PX4 instance per drone, all sharing the same Gazebo world, each spawned at its own point from the scenario, with its own MAVLink heartbeat and its own MAVROS bridge in a separate namespace. A second script, run_multi_mission.py, starts one mission node and one collision monitor per drone, waits until every drone has either reached its target or been destroyed, and writes a summary for the whole run.

The collision monitor now also checks each drone against the others. Every monitor publishes its own drone’s position in world coordinates, so each one can see where the others are. A drone that has been destroyed keeps publishing its last position, because the wreck stays where it fell and is still an obstacle for the rest.

Getting three drones to fly took much longer than expected. For a while it looked like the virtual machine was too slow, because the PX4 instances reported accelerometer timeouts and the drones fell without touching anything. The actual cause was the MAVLink system identifiers. PX4 gives instance i the MAVLink system identifier i+1, while MAVROS sends its commands to system 1 unless it is told otherwise, so all three bridges were talking to the first drone. The other two logged “Ignore command 400 from 1/191 to 1/1” and could never arm, and the commands that went unanswered were retried until two replies arrived at once and MAVROS crashed. Giving each MAVROS the system identifier of its own autopilot fixed it.

Two smaller problems came out of the same work. MAVROS cannot be used to set parameters when several drones are running, because it downloads the autopilot’s whole parameter list before it will set anything, and with three instances that download does not finish. The parameters are now sent directly over MAVLink by the heartbeat process instead. And the mission time is now measured from Gazebo’s clock rather than the wall clock, so a machine that cannot keep up produces a longer run in real time but the same simulated result.

With those fixed, three drones fly correctly. In a maze of ten obstacles all three reached their targets, with a summed Euclidean distance of 49.34 m, a summed travelled distance of 54.31 m, a ratio of 1.101 and a simulation time of 11.5 s. While they were flying, Gazebo ran at 0.99 of real time and used about 40 per cent of one of the six processor cores, so the machine was fast enough.

## Experiments over Obstacle Count and Drone Count

run_batch_3d.py runs the whole experiment without supervision. For every combination of obstacle count, drone count and random seed it generates the scenario, starts the simulator and the drones, spawns the obstacles, flies the mission, records the results and shuts everything down before the next run. compute_metrics_3d.py then collects all the runs into one table.

I ran n = 4, 8, 12, 16 and 20 obstacles with k = 1, 2 and 3 drones and five random seeds for each combination, which is 75 runs. It took 99 minutes and every run completed. The table below shows the averages for each configuration; the full results are in the repository under results/2026-09-06_3d_sweep.

n   k   reached   coll   ratio  ratio+   cpu(s)

4   1       3/5    0.4   0.932   1.097       15

4   2     10/10    0.0   1.097   1.097       32

4   3      9/15    1.2   0.954   1.097       52

8   1       3/5    0.4   0.852   1.103       14

8   2      6/10    0.8   0.865   1.099       30

8   3      9/15    1.2   0.905   1.098       50

12   1       2/5    0.6   0.733   1.095       15

12   2      4/10    1.2   0.736   1.100       33

12   3      8/15    1.4   0.848   1.104       54

16   1       1/5    0.8   0.599   1.088       13

16   2      4/10    1.2   0.740   1.102       34

16   3      6/15    1.8   0.748   1.106       49

20   1       1/5    0.8   0.602   1.086       13

20   2      3/10    1.4   0.699   1.103       30

20   3      5/15    2.0   0.733   1.110       50

Collisions per run go up with the number of obstacles at every drone count, and the share of drones that reach their target goes down. Since there is no avoidance in these runs, this is the baseline that the avoidance component has to be compared against.

There are two ratio columns. The first is over all the drones. It falls from 0.93 to 0.60 as the maze gets denser, because a drone that is destroyed halfway counts the distance it managed against the full distance it was asked to fly. The second is over only the drones that arrive, and it stays between 1.086 and 1.110 in all fifteen configurations, because without avoidance the drones fly almost in a straight line. With avoidance this second ratio should go up.

The CPU time depends on the number of drones and not on the number of obstacles, at about 14, 32 and 50 seconds per run for one, two and three drones.

Collisions between two drones happened as well, at four obstacles with three drones and at eight obstacles with three drones, but not in the denser mazes. With few obstacles the drones fly straighter and cross each other more often, while in a dense maze they are usually destroyed by an obstacle before they can meet.

The scenario generator draws the drone start and target points from the seed before it places the obstacles, so the same seed gives the same missions at every obstacle count. The summed Euclidean distance is therefore identical down each column, 21.5, 37.6 and 53.3 m for one, two and three drones, and only the obstacle density changes between the rows.

## Avoidance with Reinforcement Learning

As the professor suggested, the avoidance component is a neural network trained with reinforcement learning. Its input is a lidar scan together with the target, and its output is the direction in which the drone should move. I used a lidar instead of a camera, because camera images have to be rendered by the simulator and rendering does not work in my virtual machine, which is the same reason the first stage needed a virtual lidar. For the training I used PPO from the Stable-Baselines3 library, which is an actor-critic method.

The network cannot be trained in Gazebo with PX4. A single flight takes about a minute, and the training needs hundreds of thousands of decisions. So it is trained in MuJoCo, using the MuJoCo-Drones-Gym package, and afterwards flown in Gazebo with PX4 without any more training. This only works if the network gets the same input in both simulators and its output means the same thing in both.

## The Lidar and the Training Environment

The lidar is computed by my own code, in the same way as the virtual lidar of the first stage but in three dimensions. It casts 82 rays: 16 directions around the drone at five elevations between -60 and 60 degrees, plus one straight up and one straight down. Each ray is intersected with the obstacle boxes, the ground and the other drones, up to a range of 10 m. Since the same function runs during the training in MuJoCo and during the flights in Gazebo, the network sees the same scan in both. I tested it against distances worked out by hand and with a geometric check over 200 random mazes. Besides the 82 distances, the network receives the direction and distance to the target, its own velocity and its altitude.

The training environment gives the drone a new random maze in every episode, made by the same generator as the Gazebo experiments with 0 to 20 obstacles, so the training mazes and the evaluation mazes come from the same distribution. The reward is the progress towards the target, minus a small cost per step, a penalty for flying within 1 m of an obstacle and a large penalty for a collision, plus a bonus for arriving. Collisions are judged with the same geometric rule as the collision monitor in Gazebo.

While building the environment I found a bug in MuJoCo-Drones-Gym. In its waypoint mode it creates a new position controller at every control step, so the controller loses its state, the attitude oscillates more and more, and the drone flips over. Hovering and climbing straight up still work, which makes the problem easy to miss. I worked around it by calling the package’s controller myself and keeping one controller for the whole episode. With this fix, a drone flying straight in MuJoCo gives the same result as in Gazebo in all 25 single-drone scenarios of the baseline: the same drones crash and the clearances are almost identical. Because of this check I think it is valid to train in one simulator and test in the other.

## Training

I trained with four environments in parallel for 2 million decisions, which took 31 minutes in the virtual machine. During the training the policy was tested regularly on 100 mazes it never trains on, and the best version was kept. In a first attempt the policy improved and then got worse again as the training went on. A learning rate that decreases during the training and a limit on how much the policy can change in one update solved this. The final policy reaches the target in 92 of the 100 test mazes in MuJoCo, where flying straight reaches 51.

## Flying the Policy in PX4

The mission node has a new option that uses the policy. After the climb it builds the same input from the scenario’s obstacles and the positions of the other drones, runs the network every 0.104 s, and sends the direction to PX4 as an offboard velocity setpoint, at the same speed as in training (1.04 m/s horizontally and 1.17 m/s vertically). Without the option the drone flies straight as before, so both cases go through the same code and the same measurements. I checked that the input built during a flight is identical to the one in training: over 5,081 decisions the largest difference was zero.

Before the experiments a system update changed MAVROS from version 2.14 to 2.16, which moved its topics to a different place, so none of the drones could arm. The nodes now look up where MAVROS publishes instead of assuming it. Because the software had changed, I also repeated the baseline sweep instead of reusing the September results.

## Experiments with and without Avoidance

Both sweeps were run on the same day with the same 75 scenarios as before: 4 to 20 obstacles, 1 to 3 drones and five seeds. The repeated baseline matches the September one: 72 of the 75 runs have the same result and the totals are the same. The other three differ only in collisions between drones, which depend on small differences in timing.

n   k            reached   collisions/run           ratio+

straight   policy  straight policy  straight policy

4   1       3/5      4/5       0.4    0.2     1.105  1.128

4   2     10/10    10/10       0.0    0.0     1.097  1.127

4   3     11/15    14/15       0.8    0.2     1.097  1.166

8   1       3/5      5/5       0.4    0.0     1.099  1.137

8   2      5/10     9/10       1.0    0.2     1.102  1.150

8   3      8/15    15/15       1.4    0.0     1.095  1.205

12   1       2/5      5/5       0.6    0.0     1.092  1.155

12   2      4/10     9/10       1.2    0.2     1.103  1.171

12   3      8/15    13/15       1.4    0.4     1.102  1.253

16   1       1/5      5/5       0.8    0.0     1.091  1.172

16   2      4/10    10/10       1.2    0.0     1.104  1.180

16   3      6/15    11/15       1.8    0.8     1.104  1.294

20   1       1/5      5/5       0.8    0.0     1.090  1.273

20   2      3/10     9/10       1.4    0.2     1.102  1.209

20   3      5/15    11/15       2.0    0.8     1.109  1.335

With the policy, 135 of the 150 drones reach their target instead of 74, so 90% instead of 49%, and the number of destroyed drones falls from 76 to 15. Comparing the same drone in the same scenario, the policy saves 61 drones and does not lose any. Every drone that arrives flying straight also arrives with the policy. The ratio over the drones that arrive is now between 1.13 and 1.34 instead of 1.09 to 1.11, and it grows with the number of obstacles, which means the drones are now flying around the obstacles instead of straight at the target. With the policy there were no collisions between drones (three without it), and no two drones came closer than 2.66 m.

The Gazebo result is also close to what MuJoCo predicted. For the 25 single-drone scenarios MuJoCo predicted 23 arrivals, and Gazebo gave 24.

## Costs and Limitations

The missions are slower, about 30 s instead of 12 s, mainly because the policy flies at the 1.04 m/s it was trained at, while PX4 flying straight averages about 1.7 m/s. The network costs about 3.5 s of CPU time per mission with one drone, and the simulation still ran in real time with three drones.

The policy fails more often when more drones are flying: 4% of the drones are destroyed with one drone, 6% with two and 15% with three. None of these crashes happened near another drone. I can think of two reasons, and these results cannot tell them apart. The first is that the mazes with three drones are a different set, because the generator places the obstacles after all the start and target points. The second is that the other drones in the lidar confuse the network, since it was trained with one drone only.

The drones that the policy saves often pass close to an obstacle, with a median closest distance of 0.45 m. The penalty for flying close to an obstacle is small compared to the penalty for a crash, so the policy learned to pass through gaps rather than to keep away from walls.